| ELE432 Advanced Digital Design Preliminary Work 3 Multicycle Processor RISC-V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name: Emre Kerim Kaymak |  |  |  | Number: 2220357045 |  |

Q1) HOURS SPENT

Approximately 5 hours has been spent on this homework, 2 hours of writing the code, 1 hour of debugging, I was quite lucky that the code worked in the first iteration this time. The remaining 2 hours were used for drawing schematics as well and also to prepare this document.

Q2) HIERARCHICAL DIAGRAM

module top(

input  logic clk, reset,

output logic [31:0] WriteData, DataAdr,

output logic MemWrite

);

// ================= INTERNAL SIGNALS =================

// PC path

logic [31:0] PC, PCNext, OldPC;

// Memory

logic [31:0] Adr, ReadData, Data;

// Instruction

logic [31:0] Instr;

// Register file

logic [31:0] RD1, RD2;

logic [31:0] RD1_reg, RD2_reg;

// ALU

logic [31:0] SrcA, SrcB, ALUResult, ALUOut;

logic Zero;

// Immediate

logic [31:0] ImmExt;

// Result

logic [31:0] Result;

// Control signals

logic [1:0] ImmSrc, ALUSrcA, ALUSrcB, ResultSrc;

logic [2:0] ALUControl;

logic AdrSrc;

logic IRWrite, PCWrite, RegWrite;

// ================= CONTROLLER =================

controller ctrl (

.clk(clk),

.reset(reset),

.op(Instr[6:0]),

.funct3(Instr[14:12]),

.funct7b5(Instr[30]),

.zero(Zero),

.immsrc(ImmSrc),

.alusrca(ALUSrcA),

.alusrcb(ALUSrcB),

.resultsrc(ResultSrc),

.adrsrc(AdrSrc),

.alucontrol(ALUControl),

.irwrite(IRWrite),

.pcwrite(PCWrite),

.regwrite(RegWrite),

.memwrite(MemWrite)

);

Q3) SYSTEM VERILOG CODE

// ================= PC REGISTER =================

always_ff @(posedge clk or posedge reset) begin

if (reset)

PC <= 32'b0;

else if (PCWrite)

PC <= PCNext;

end

// ================= OLD PC REGISTER =================

always_ff @(posedge clk) begin

if (IRWrite)

OldPC <= PC;

end

// ================= ADDRESS MUX =================

assign Adr = (AdrSrc) ? Result : PC;

// ================= MEMORY =================

mem memory (

.clk(clk),

.WE(MemWrite),

.A(Adr),

.WD(RD2_reg),

.RD(ReadData)

);

// ================= INSTRUCTION REGISTER ================

always_ff @(posedge clk) begin

if (IRWrite)

Instr <= ReadData;

end

// ================= DATA REGISTER =================

always_ff @(posedge clk) begin

Data <= ReadData;

end

// ================= REGISTER FILE =================

regfile rf (

.clk(clk),

.reset(reset),

.WE3(RegWrite),

.A1(Instr[19:15]),

.A2(Instr[24:20]),

.A3(Instr[11:7]),

.WD3(Result),

.RD1(RD1),

.RD2(RD2)

);

// ================= RD1 / RD2 PIPE REGISTERS =================

always_ff @(posedge clk) begin

RD1_reg <= RD1;

RD2_reg <= RD2;

end

// ================= IMMEDIATE =================

sign_extend imm (

.instr(Instr),

.Immsrc(ImmSrc),

.dataout(ImmExt)

);

// ================= SRC A MUX =================

always_comb begin

case (ALUSrcA)

2'b00: SrcA = PC;

2'b01: SrcA = OldPC;

2'b10: SrcA = RD1_reg;

default: SrcA = 32'b0;

endcase

end

// ================= SRC B MUX =================

always_comb begin

case (ALUSrcB)

2'b00: SrcB = RD2_reg;

2'b01: SrcB = ImmExt;   // ← your correction

2'b10: SrcB = 32'd4;    // ← your correction

default: SrcB = 32'b0;

endcase

end

// ================= ALU =================

myALU alu (

.SrcA(SrcA),

.SrcB(SrcB),

.ALUControl(ALUControl),

.zerof(Zero),

.ALUResult(ALUResult)

);

// ================= ALU OUTPUT REGISTER =================

always_ff @(posedge clk) begin

ALUOut <= ALUResult;

end

// ================= RESULT MUX =================

always_comb begin

case (ResultSrc)

2'b00: Result = ALUOut;

2'b01: Result = Data;

2'b10: Result = ALUResult;

default: Result = 32'b0;

endcase

end

// ================= PC NEXT =================

assign PCNext = Result;

// ================= OUTPUTS =================

assign WriteData = RD2_reg;

assign DataAdr   = Adr;

endmodule

module regfile (

input  logic        clk,

input  logic        reset,

input  logic        WE3,

input  logic [4:0]  A1,

input  logic [4:0]  A2,

input  logic [4:0]  A3,

input  logic [31:0] WD3,

output logic [31:0] RD1,

output logic [31:0] RD2

);

logic [31:0] rf [31:0];

always_ff @(posedge clk or posedge reset) begin

if (reset) begin

for (int i = 0; i < 32; i++)

rf[i] <= 32'b0;

end

else if (WE3 && (A3 != 5'b0)) begin

rf[A3] <= WD3;

end

end

always_comb begin

RD1 = (A1 == 5'b0) ? 32'b0 : rf[A1];

RD2 = (A2 == 5'b0) ? 32'b0 : rf[A2];

end

endmodule

module sign_extend (

input  logic [31:0] instr,

input  logic [1:0]  Immsrc,

output logic [31:0] dataout

);

always_comb begin

case (Immsrc)

2'b00:  // I-type

dataout = {{20{instr[31]}}, instr[31:20]};

2'b01:  // S-type

dataout = {{20{instr[31]}}, instr[31:25], instr[11:7]};

2'b10:  // B-type

dataout = {{20{instr[31]}}, instr[7], instr[30:25], instr[11:8], 1'b0};

2'b11:  // J-type

dataout = {{12{instr[31]}}, instr[19:12], instr[20], instr[30:21], 1'b0};

default:

dataout = 32'b0;

endcase

end

endmodule

module controller(

input  logic clk,

input  logic reset,

input  logic [6:0] op,

input  logic [2:0] funct3,

input  logic funct7b5,

input  logic zero,

output logic [1:0] immsrc,

output logic [1:0] alusrca, alusrcb,

output logic [1:0] resultsrc,

output logic adrsrc,

output logic [2:0] alucontrol,

output logic irwrite, pcwrite,

output logic regwrite, memwrite

);

// ========= OPCODE CONSTANTS =========

localparam [6:0]

LW  = 7'b0000011,

SW  = 7'b0100011,

RT  = 7'b0110011,

IT  = 7'b0010011,

JAL = 7'b1101111,

BEQ = 7'b1100011;

// ========= STATE TYPE =========

typedef enum logic [3:0] {

FETCH,

DECODE,

MEMADR,

MEMREAD,

MEMWB,

MEMWRITE,

EXECUTER,

ALUWB,

EXECUTEL,

JAL_S,

BEQ_S

} state_t;

state_t state_cur, state_next;

logic [1:0] ALUOp;

// ALU DECODER

aludec alu_decoder (

.opb5(op[5]),

.funct3(funct3),

.funct7b5(funct7b5),

.ALUOp(ALUOp),

.ALUControl(alucontrol)

);

//INSTR DECODER

instrdec instr_decoder (

.op(op),

.ImmSrc(immsrc)

);

// ================= STATE REGISTER =================

always_ff @(posedge clk or posedge reset) begin

if (reset)

state_cur <= FETCH;

else

state_cur <= state_next;

end

// ================= NEXT STATE LOGIC =================

always_comb begin

state_next = state_cur;

case (state_cur)

FETCH:    state_next = DECODE;

DECODE: begin

case (op)

LW, SW: state_next = MEMADR;

RT:     state_next = EXECUTER;

IT:     state_next = EXECUTEL;

JAL:    state_next = JAL_S;

BEQ:    state_next = BEQ_S;

default:state_next = FETCH;

endcase

end

MEMADR: begin

if (op == LW) state_next = MEMREAD;

else          state_next = MEMWRITE;

end

MEMREAD:   state_next = MEMWB;

MEMWB:     state_next = FETCH;

MEMWRITE:  state_next = FETCH;

EXECUTER:  state_next = ALUWB;

EXECUTEL:  state_next = ALUWB;

BEQ_S:     state_next = FETCH;

ALUWB:     state_next = FETCH;

JAL_S:     state_next = ALUWB;

endcase

end

// ================= OUTPUT LOGIC =================

always_comb begin

// ===== DEFAULTS =====

//immsrc    = 2'b00;

alusrca   = 2'b00;

alusrcb   = 2'b00;

resultsrc = 2'b00;

adrsrc    = 1'b0;

irwrite   = 1'b0;

pcwrite   = 1'b0;

regwrite  = 1'b0;

memwrite  = 1'b0;

ALUOp     = 2'b00;

/*

// ===== IMM SRC based on opcode =====

case (op)

LW, IT: immsrc = 2'b00;

SW:     immsrc = 2'b01;

BEQ:    immsrc = 2'b10;

JAL:    immsrc = 2'b11;

default: immsrc = 2'b00;

endcase

*/

// ===== STATE-BASED OUTPUTS =====

case (state_cur)

FETCH: begin

alusrca = 2'b00;

alusrcb = 2'b10;

irwrite = 1'b1;

pcwrite = 1'b1;

resultsrc = 2'b10;

adrsrc = 1'b0;

ALUOp = 2'b00;

end

DECODE: begin

alusrca = 2'b01;

alusrcb = 2'b01;

ALUOp = 2'b00;

end

MEMADR: begin

alusrca = 2'b10;

alusrcb = 2'b01;

ALUOp = 2'b00;

end

MEMREAD: begin

adrsrc = 1'b1;

resultsrc = 2'b00;

end

MEMWB: begin

resultsrc = 2'b01;

regwrite = 1'b1;

end

MEMWRITE: begin

resultsrc = 2'b00;

adrsrc = 1'b1;

memwrite = 1'b1;

end

EXECUTER: begin

alusrca = 2'b10;

alusrcb = 2'b00;

ALUOp = 2'b10;

end

EXECUTEL: begin

alusrca = 2'b10;

alusrcb = 2'b01;

ALUOp = 2'b10;

end

ALUWB: begin

resultsrc = 2'b00;

regwrite = 1'b1;

end

JAL_S: begin

alusrca = 2'b01;

alusrcb = 2'b10;

ALUOp = 2'b00;

resultsrc = 2'b00;

pcwrite = 1'b1;

end

BEQ_S: begin

alusrca = 2'b10;

alusrcb = 2'b00;

ALUOp = 2'b01;

resultsrc = 2'b00;

if (zero)

pcwrite = 1'b1;

end

endcase

end

endmodule

module aludec(input logic opb5,

input logic [2:0] funct3,

input logic funct7b5,

input logic [1:0] ALUOp,

output logic [2:0] ALUControl);

logic RtypeSub;

assign RtypeSub = funct7b5 & opb5; // TRUE for R-type subtract instruction

always_comb

case(ALUOp)

2'b00: ALUControl = 3'b000; // addition

2'b01: ALUControl = 3'b001; // subtraction

default: case(funct3) // R-type or I-type ALU

3'b000: if (RtypeSub)

ALUControl = 3'b001; // sub

else

ALUControl = 3'b000; // add, addi

3'b010: ALUControl = 3'b101; // slt, slti

3'b110: ALUControl = 3'b011; // or, ori

3'b111: ALUControl = 3'b010; // and, andi

default: ALUControl = 3'bxxx; // ???

endcase

endcase

endmodule

module instrdec(

input logic [6:0] op,

output logic [1:0] ImmSrc

);

always_comb

case(op)

7'b0110011: ImmSrc = 2'b00; // R-type

7'b0010011: ImmSrc = 2'b00; // I-type ALU

7'b0000011: ImmSrc = 2'b00; // lw

7'b0100011: ImmSrc = 2'b01; // sw

7'b1100011: ImmSrc = 2'b10; // beq

7'b1101111: ImmSrc = 2'b11; // jal

default: ImmSrc = 2'b00;

endcase

endmodule

module mem (

input  logic        clk,

input  logic        WE,       // write enable

input  logic [31:0] A,        // address

input  logic [31:0] WD,       // write data

output logic [31:0] RD        // read data

);

logic [31:0] RAM [0:63];

// Initialize memory from file

initial begin

$readmemh("memfile.txt", RAM);

end

// Write (synchronous)

always_ff @(posedge clk) begin

if (WE) begin

RAM[A[31:2]] <= WD;

end

end

// Read (asynchronous)

assign RD = RAM[A[31:2]];

endmodule

module adder_n #(

parameter N = 32

) (

input  logic [N-1:0] a,

input  logic [N-1:0] b,

input  logic         cin,

output logic [N-1:0] sum,

output logic         cout

);

assign {cout, sum} = a + b + cin;

endmodule

module myALU #(

parameter N = 32

)(

input  logic [31:0] SrcA,

input  logic [31:0] SrcB,

input  logic [2:0]  ALUControl,

output logic        zerof,

output logic [31:0] ALUResult

);

logic [31:0] Bmux;

logic [31:0] sum;

logic        cout;

logic        overflow;

logic        cin;

assign Bmux = SrcB ^ {32{ALUControl[0]}}; //basically take invert if 1 o.w. 0

assign cin  = ALUControl[0];

adder_n #(.N(32)) adder_inst (

.a   (SrcA),

.b   (Bmux),

.cin (cin),

.sum (sum),

.cout(cout)

);

assign overflow =

(ALUControl[1] == 1'b0) &&

(SrcA[31] == Bmux[31]) &&

(sum[31]  != SrcA[31]);

// ALU Operations

always_comb begin

case (ALUControl)

3'b000: ALUResult = sum;                      // ADD

3'b001: ALUResult = sum;                      // SUB

3'b010: ALUResult = SrcA & SrcB;              // AND

3'b011: ALUResult = SrcA | SrcB;              // OR

3'b100: ALUResult = SrcA ^ SrcB;              // XOR

3'b101: // SLT (signed)

ALUResult = {31'b0, (overflow ^ sum[31])};

3'b110: ALUResult = SrcA >> SrcB[4:0];        // SRL

3'b111: ALUResult = SrcA << SrcB[4:0];        // SLL

default: ALUResult = 32'b0;

endcase

zerof = (ALUResult == 32'b0);

end

endmodule

Q4) KEY SIGNALS FOR FIRST 3 INSTRUCTIONS

| STEP | PC | INSTR | STATE | RESULT (in hex) |
| --- | --- | --- | --- | --- |
| 3 | 00 | 00500113 | FETCH | 4 |
| 4 | 04 | 00500113 | DECODE | X |
| 5 | 04 | 00500113 | EXECUTEL | X |
| 6 | 04 | 00500113 | ALUWB | 5 |
| 7 | 04 | 00500113 | FETCH | 8 |
| 8 | 08 | 00C00193 | DECODE | X |
| 9 | 08 | 00C00193 | EXECUTEL | X |
| 10 | 08 | 00C00193 | ALUWB | C |
| 11 | 08 | 00C00193 | FETCH | C |
| 12 | 0C | FF718393 | DECODE | X |
| 13 | 0C | FF718393 | EXECUTEL | X |
| 14 | 0C | FF718393 | ALUWB | 3 |
| 15 | 10 | FF718393 | FETCH | 10 |

Q5) SIMULATION WAVEFORMS

At the end of the last page, the confirmation that the test vector is correct can be checked, as given is the riscvtest.s file “# If successful, it should write the value 25 to address 100”, here it can clearly be seen that this is correct.

A zoomed in part of the page is given:

A slight problem is that every instruction seem to happen a PC in delay meaning at a  PC + 4 of what is the required by the file. Otherwise, no other serious problem can be observed.

Q6) GITHUB LINK

https://github.com/TM-LEONARDO/RISC_V-MULTICYCLE-PROCESSOR_-PRELIMINARY/tree/main